Milestone A — Get waypoint-following actually working

Why this matters: Nothing else can happen until the base can reliably drive itself around the field. This isn't new work, it's finishing the debugging thread that was already open.

Steps:

- Open mecanum_controller.py, find the last two lines of the control loop (the ones computing forward_speed and omega), and replace them with the untested fix:

python

OMEGA_MAX = 1.5

omega = max(-OMEGA_MAX, min(OMEGA_MAX, K_HEADING * heading_error))

forward_speed = min(MAX_SPEED, dist) * max(0.0, math.cos(heading_error))

- Save the controller file (this is a separate save from the world file — Ctrl+S in whatever editor you're using for the .py, and Ctrl+S in Webots for the .wbt if you touched the Scene Tree).

- In Webots: Reset (⟲), then Real-time (▶).

- Watch the console for [Nav] Reached waypoint X/28 messages and watch the 3D view.

What "working" looks like: the robot rotates to roughly face the first waypoint, drives toward it without excessive spinning, reaches it, and the message count climbs at a reasonable pace — not 40 seconds per waypoint like before.

If it still fails: don't guess a fifth fix blind. Add a debug print inside the loop:

python

print(f"dist={dist:.2f} heading_err={math.degrees(heading_error):.1f} fwd={forward_speed:.2f} omega={omega:.2f}")

Run for a few seconds, look at the numbers. If forward_speed stays near zero even when heading_error is small, the taper math is wrong. If omega is pinned at OMEGA_MAX constantly, K_HEADING is too aggressive or the sign is backwards. This turns "it's still broken" into an actual diagnosis instead of another guess.

Done when: one full 28-waypoint pass completes without the robot getting stuck, oscillating, or overshooting the field boundary.

Milestone B — Full coverage run with logging (→ your Figure 6a)

Why this matters: A visual "it drove around" isn't a result. A paper needs a plotted trajectory with numbers behind it — this is the first piece of real data replacing the paper template's fabricated Figure 6a.

Steps:

- In the controller's main loop, every timestep (or every N steps to keep the file smaller — every ~0.5s is plenty), append (time, x, y, heading) from GPS + IMU to a list.

- When the mission ends (all waypoints reached), write that list to a CSV — e.g. results/trajectory_log.csv. You already have a habit of writing to results/ from the delta-robot code, so mirror that pattern here for consistency.

- Outside Webots, in a normal Python/matplotlib script (not inside the controller — controllers should stay lightweight), read the CSV and plot:

  - the 10×10m field boundary as a rectangle

  - the 28 planned waypoints as an overlay path (dashed line or markers)

  - the actual GPS-logged trajectory as a solid line on top

- Also compute simple summary numbers: total path length driven, total mission time, average deviation between planned waypoint line segments and actual position at the closest matching time.

What good output looks like: the actual trajectory should roughly trace the serpentine pattern but with visible wobble, overshoot at turns, and settling behavior near each waypoint — because that's what a real proportional controller on real (simulated) physics does. Don't be alarmed if it's not a razor-straight line; that imperfection is itself evidence this is a real simulation and not the paper template's clip-art-perfect version.

Common trap: logging every single physics timestep (TIME_STEP = 32 ms) for a multi-minute run will produce tens of thousands of rows — fine for a CSV, but downsample when plotting or your figure will be a solid blob. Plot every 5th–10th sample.

Done when: you have trajectory_log.csv and a saved figure showing planned vs. actual path over the full run.

Milestone C — Real localization error data (→ your Figure 6b)

Why this matters: This is the piece that most obviously separates you from the paper template. The template's EKF section literally feeds ekf.update_imu(0.0, 0.0, 0.0) — hardcoded zeros — and fakes GPS noise with np.random.normal. In Webots, your GPS/IMU nodes are near ground-truth by default, so you have to deliberately inject realistic noise and then show your system handling it, rather than just reading a perfect sensor.

Steps:

- Decide what "ground truth" means in Webots. Webots itself knows the exact simulated position of the robot at all times — that's your ground truth. Your GPS reading is currently basically equal to it (maybe with Webots' own tiny built-in sensor noise, check the GPS node's noise field if it exists in R2025a).

- Inject noise deliberately. Two options:

  - Simplest: in the controller, take the raw gps.getValues() and add np.random.normal(0, sigma) to x and y yourself before using it for control/logging, where sigma is something like 0.02–0.05m to mimic real RTK-GPS-ish behavior.

  - More realistic: check if the GPS node in Webots has a noise field you can set directly in the Scene Tree instead of doing it in Python.

- Build a minimal filter, even something much simpler than a full 6-state EKF — a basic complementary filter or a simplified Kalman filter fusing noisy-GPS position with IMU-yaw-integrated dead reckoning is enough to generate a real "error over time" story. This can be a lightweight port of the EKF logic sitting unused in mobile_robot_navigation.py, but rewired to consume actual per-timestep Webots sensor values instead of the one-shot faked call in that file.

- Log three things per timestep: ground-truth position (from Webots directly, if accessible — otherwise treat unnoised GPS as your proxy ground truth), noisy raw GPS reading, and filtered/estimated position.

- Simulate a "GPS blackout" period to mirror the paper's canopy-shadowing claim: for a stretch of the run (say 30–60 seconds), stop feeding GPS updates into your filter entirely (or don't add new GPS-based corrections) and let it run on IMU-only dead reckoning, then resume GPS. This directly gives you the "degrades gracefully, then recovers" story instead of you inventing that claim.

- Plot error (distance between estimate and ground truth) vs. time, with the blackout window shaded.

Scope-check: you don't need a textbook-perfect 6-state EKF to make this legitimate — you need real noise, real filtering, and real logged error, even if the filter is simpler than the paper's fictional one. Note in your own writeup exactly what filter you actually used; don't claim a 6-state EKF if you built a 2-state complementary filter.

Done when: you have a plotted error-vs-time curve with real numbers (mean error, degraded-period error) computed from logged data, not typed in by hand.

Milestone D — Crab-walk vs. turn comparison (your original contribution)

Why this matters: This doesn't exist anywhere in the paper template or the original pure-Python code — it's the one piece of genuinely new science in this project, and it's straightforward to generate cleanly in Webots because you already have both motion modes physically working (holonomic crab motion via the anisotropic-friction trick, and normal driving/turning).

Steps:

- Add a mode variable, not two separate controller files — e.g. ROW_SWITCH_MODE = "crab" or "turn" at the top of the controller, toggled manually between runs. Two files would drift out of sync the moment you fix a bug in one and forget the other; you already learned this lesson with the config.py consolidation on the delta-robot side, apply it here too.

- Implement the two row-switch behaviors at the point in your Boustrophedon logic where the robot reaches the end of a stripe and needs to move to the next row:

  - Crab mode: command vy directly (pure lateral motion, omega ≈ 0) to shift over by stripe_width without changing heading.

  - Turn mode: stop forward motion, rotate ~180° in place (command omega until heading changes by π), then drive forward into the next row.

- Instrument the switch maneuver specifically — not the whole mission, just the transition between finishing one row and starting the next stripe. Record: start time, end time (when the robot is aligned and moving into the new row), and per-timestep wheel velocity commands during that window.

- Estimate energy. You don't have real torque sensors, so use a physically reasonable proxy consistently applied to both modes — e.g. sum |wheel_velocity| × dt per wheel as a work proxy, or if you want something a bit more physical, multiply by an assumed constant torque-per-unit-load factor (similar in spirit to the ACTUATOR_J/ACTUATOR_B estimates you already used in config.py for the delta arm — reuse that "flag it as an engineering estimate, not measured" approach here too).

- Run the full field multiple times per mode (3–5 repeats each) so you get a mean ± std instead of one anecdotal number — this is what makes it a result rather than a demo.

- Collect into a CSV: mode, trial, switch_time_s, energy_estimate. Compute mean/std per mode, and optionally a plot (grouped bar chart, crab vs. turn, time and energy side by side — similar style to the bar charts already in your delta-robot robustness_analysis.py, which you can basically reuse the plotting pattern from).

Done when: you have a table like "Crab: mean switch time X ± Y s, energy Z ± W (units)" vs. the same for Turn, from real repeated Webots trials — this alone is a legitimate, novel results subsection.

Milestone E — Obstacle avoidance (decide scope now, don't defer to the end)

Why this matters: The paper template claims VFH+ with LiDAR obstacle avoidance. Nothing resembling that exists in your Webots world yet — no LiDAR device, no avoidance logic. Left undecided, this becomes a claim in your paper with zero code behind it, discovered only at write-up time.

Two honest paths, pick one deliberately:

- Build a minimal version. Add a Webots Lidar (or even a simpler distance-sensor ring, which is much less work) node to the chassis, and write a simple reactive avoidance behavior — e.g. if an obstacle is detected within some threshold in the direction of travel, reduce forward speed or bias omega away from it. It does not need to be true VFH+; a documented simplified reactive avoidance is fine as long as you describe it accurately.

- Scope it out. Explicitly decide the final paper will not claim LiDAR/VFH+ obstacle avoidance, and rewrite that section to describe only what you actually built (GPS+IMU waypoint navigation, no dynamic obstacle handling). This is a perfectly reasonable scoping decision for a project already this large.