Autonomous Weed Removal Robot (AWRR)
Daily Technical Summary & Comparative Benchmarks

Omnidirectional Mecanum Drive vs. Front-Wheel Ackermann Steering Benchmark (16m × 16m Arena, 9 Crop Rows)
Workspace: C:\Users\User\Documents\git_abd | Date: August 25, 2026

# 1. Workspace Consolidation & Repository Setup

To eliminate confusion caused by multiple duplicate project folders across Desktop and Documents, all codebase files, Webots simulations, kinematics models, research PDFs, and documentation were consolidated into a single unified directory: C:\Users\User\Documents\git_abd. The .gitignore UTF-16LE encoding conflict was resolved to standard UTF-8 without BOM to ensure clean Git operations.

# 2. Kinematics Architecture: Ackermann vs. Mecanum Drive

Both drive architectures were implemented and benchmarked on identical 16m × 16m agricultural field setups:

| Drive Architecture | Front-Wheel Ackermann Steering | 4-Wheel Independent Mecanum Drive |
| --- | --- | --- |
| Steering Mechanics | 2 Front Steering Motors [-44.5°, +44.5°], Fixed Rear Axle | 0 Steering Motors (Differential Wheel Speeds ω_fl, ω_fr, ω_rl, ω_rr) |
| Turning Geometry | Semicircular Dubins Headland Arcs (Radius R = 0.8m) | Zero-Radius In-Place Rotation & Lateral Crab-Strafing |
| Actuator Complexity | 4 Drive Motors + 2 Steering Motors | 4 Drive Motors Only |

# 3. Webots 3D Environment & Field Layout Upgrades

Two dedicated world files were constructed in Webots: WEEDBOT_ACKERMANN.wbt and WEEDBOT_MECANUM.wbt. Both worlds share identical 16m × 16m floor dimensions, 0.40m perimeter boundary walls at ±8.0m, 9 visual green crop row lines at 1.6m spacing (Y = -6.4m to +6.4m), and 2.5m dark brown headland turning strips.

# 4. Key Navigation Bug Fixes & Algorithms

Donut Loop Resolution: Re-parametrized U-turn arcs with exact semicircular Dubins equations (theta in [0, pi]), eliminating 360° teardrop spinning traps.

Vector Projection Advancement: Implemented forward dot-product check (dx·cos θ + dy·sin θ < 0). If a waypoint falls behind the front axle, the controller automatically advances to the next point.

Initial Placement Alignment: Set robot starting translation to (-5.0m, -6.4m, 0.15m) facing East, centering the robot perfectly on Row 1 centerline.

1.30m Physical Wall Clearance: Headland U-turn arc peaks reach X = ±5.8m, leaving 1.30m physical clearance from perimeter walls at ±8.0m.

# 5. Comparative Benchmark Results & Generated Artifacts

| Performance Metric | Ackermann Steering Platform | Mecanum Omnidirectional Platform |
| --- | --- | --- |
| World Simulation File | WEEDBOT_ACKERMANN.wbt | WEEDBOT_MECANUM.wbt |
| Python Controller Script | ackermann_steering_controller.py | mecanum_steering_controller.py |
| Field Surface Coverage Efficiency | 90% (14.4m / 16.0m) | 90% (14.4m / 16.0m) |
| Total Waypoints Completed | 90 / 90 Waypoints (100%) | 90 / 90 Waypoints (100%) |
| Pictorial Results Plot | figure_WEEDBOT_ACKERMANN_16m_9row_...png | figure_WEEDBOT_MECANUM_16m_9row_...png |
| Comparative Benchmark Figure | figure_COMPARATIVE_MECANUM_VS_ACKERMANN_16m_9row.png | figure_COMPARATIVE_MECANUM_VS_ACKERMANN_16m_9row.png |